Table 7.6.2.3.

| Components | Fault Code | Monitor Strategy | Fault detection criteria | MI activation criteria | Secondary Parameters | Pre-conditioning | Demonstration test |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Vehicle speed sensor | P0A3F | Hall Signal integrity | Hall Signal abnormal | / | / | N.A. | One type I |
| Vehicle speed sensor | P0A44 | Motor overspeed fault | Motor speed is higher than the specified value | / | / | N.A. | One type I |
| Drive Motor temperature sensor | P0A2A | MCU The line voltage of the temperature sensor is too high | MCU Temperature fault | / | / | N.A. | One type I |
| Voltage sensor | P0AFA | The power voltage is too low | The power voltage＜60V | / | / | N.A. | One type I |
| Voltage sensor | P0AFB | The power voltage is too high | The power voltage＞85V | / | / | N.A. | One type I |
| Current sensor | P0A51 | The power over current | The power dynamic current＞60A（single） | / | / | N.A. | One type I |
| Motor error | P0A0F | Motor gridlock ,Full open motor accelerator | Motor gridlock fault | / | / | N.A. | One type I |
| Lost Communication /MCU | U0110 | CAN Communication failure | Signal anomaly | / | / | N.A. | One type I |
| Throttle（accelerator） | P0122 | Accelerator handle fault | Abnormal speed regulation | / | / | N.A. | One type I |
| Voltage sensor battery | P1005 | The battery voltage is too low | The battery voltage＜60V | / | / | N.A. | One type I |
| Voltage sensor battery | P1004 | The battery voltage is too high | The battery voltage＞85V | / | / | N.A. | One type I |
| Temperature sensor battery | P1000 | High battery temperature | The battery temperature＞60° | / | / | N.A. | One type I |
| Temperature sensor battery | P1001 | Low battery temperature | The battery temperature＜-20° | / | / | N.A. | One type I |
| Current sensor battery | P1103 | Charge overcurrent | Charging current＞20A（single） | / | / | N.A. | One type I |
| Contactor battery | P100A | Battery case conduction | Insulation resistance＞10MΩ |  |  |  |  |
| Communications module battery | U1000 | Battery CAN Communication failure | Signal anomaly |  |  |  |  |
| Temperature sensor battery | P0A9B | Cell temperature | The temperature difference between cells is too large |  |  |  |  |